TECHNICAL MANUAL

LSTM Hyperparameter Selection for River Water Level Forecasting

A Step-by-Step Decision Framework Based on Basin Physics and Data Analysis

Applied to: Brahmaputra-Jamuna and Feni/Muhuri Rivers, Bangladesh

Model Architecture: Single-Step Routing-Aware LSTM

Version 1.0  |  2025

About This Manual This manual document the scientific and data-driven basis for every hyperparameter and feature-engineering decision in the routing-aware LSTM water level model. It explains how basin physics, cross-correlation analysis, and autocorrelation analysis were used — step by step — to arrive at each value. All methodology is grounded in peer-reviewed hydrology and machine-learning literature. Key references: Nearing et al. (2024), Kratzert et al. (2019), Hochreiter & Schmidhuber (1997), Sivapalan et al. (2003), Beven (2012).

# 1.  Overview of the Modelling Framework

## 1.1 Model Architecture

The model used is a single-step Long Short-Term Memory (LSTM) neural network (Hochreiter & Schmidhuber, 1997) trained to predict the water level on the next day, given a fixed-length window of past meteorological and hydrological features. This is a simulation model, not a direct forecast model: it is trained on ERA5 reanalysis data where all inputs are known. For real-time forecasting, HRES operational forecasts replace the ERA5 inputs, and the model is rolled forward 15 times to produce a 15-day ahead forecast.

Why single-step prediction rather than encoder-decoder? The features include autoregressive water level lags (wl_lag1, wl_lag2, ...). In an encoder-decoder multi-step architecture, the decoder would need to feed its own predictions back as lag features at steps 2-15. This compounds errors rapidly — small errors at step 2 become large errors by step 15. A single-step model avoids this by predicting only one day at a time. At HRES inference time, the window is rolled forward one day at a time, with each predicted WL inserted into the history for the next step. This is equivalent to an unrolled RNN over the 15-day horizon, but with explicit control over error propagation. Reference: Boulmaiz et al. (2020) demonstrated that autoregressive single-step LSTM outperforms encoder-decoder for flood peak prediction in small basins.

## 1.2 The HRES Lead Time Concept

During ERA5 training there is no forecast lead time — the model predicts day T+1 using inputs from days T-89 to T, all of which are known reanalysis values. The concept of lead time only applies at inference time when HRES forecast data is used:

| Lead Time | Meaning |
| --- | --- |
| Lead 1 | Predict tomorrow's WL using HRES day-1 precipitation, temperature, radiation |
| Lead 2 | Predict WL two days from now using HRES day-2 inputs + predicted WL from lead-1 |
| Lead 5 | Predict WL five days from now — moderate uncertainty |
| Lead 15 | Predict WL fifteen days from now — highest uncertainty |

The key insight from Nearing et al. (2024) is that AI-based models can maintain useful skill at 5-day lead times that equals or exceeds the skill of traditional models at 0-day lead time (nowcast), providing additional days of actionable flood warning.

# 2.  Step-by-Step Decision Framework

The following eight decisions must be made before training any LSTM model on a new river basin. Each decision depends on basin-specific physics and data analysis, not arbitrary choices. The steps must be performed in the order shown because later decisions depend on the results of earlier ones.

Decision Order (must be followed sequentially) Step 1: Compute WL autocorrelation → informs WL lag selection and HINDCAST_LEN Step 2: Compute cross-correlation of rolling precipitation vs WL → informs rolling window sizes and HINDCAST_LEN Step 3: Compute cross-correlation of temperature vs WL → informs t2m lag and snowmelt/ET decision Step 4: Assess basin physics → confirms window choices, informs anomaly window Step 5: Set HINDCAST_LEN → based on Steps 1-4 Step 6: Select all feature windows → based on Steps 1-4 Step 7: Set model architecture (HIDDEN_SIZE, DROPOUT) → based on basin complexity and data size Step 8: Set training parameters (LR, PATIENCE, BATCH_SIZE) → general best practices

## Step 1: WL Autocorrelation Analysis

Compute the Pearson correlation between the water level time series and a lagged version of itself at lags 1 through 60 days. This answers: how many days of past WL still carries information about today's WL?

# Python code to compute WL autocorrelation from scipy.stats import pearsonr import pandas as pd wl = df['water_level'].dropna() for lag in [1, 2, 3, 5, 7, 10, 14, 21, 30, 60]: r, _ = pearsonr(wl[lag:].values, wl[:-lag].values) print(f'lag {lag:>2}d : r = {r:.4f}')

### Results for Brahmaputra-Jamuna (from your data)

| Lag | Pearson r | Interpretation |
| --- | --- | --- |
| 1 day | r = 0.9985 | Extremely strong — yesterday's WL predicts today's almost perfectly |
| 2 days | r = 0.9949 | Still very strong |
| 3 days | r = 0.9900 | Strong |
| 7 days | r = 0.9665 | Strong — justifies wl_lag7 |
| 14 days | r = 0.9260 | Significant — justifies wl_lag14 |
| 21 days | r = 0.8815 | Moderate |
| 30 days | r = 0.8097 | Moderate |
| 60 days | r = 0.4727 | Weak — memory nearly lost at 60 days |

The autocorrelation drops below 0.50 near 60 days, which directly sets the upper bound on how far back WL lags are useful. The lags selected (1, 2, 3, 7, 14) cover the range where autocorrelation is still above 0.90, sampling the decay at key intervals.

### What to look for when applying to a new basin

- If autocorrelation drops below 0.90 at lag-7 (small, flashy basin): use shorter lags (1, 2, 3, 5, 7)

- If autocorrelation is still above 0.95 at lag-14 (large slow basin): include lags up to 21 or 30

- The lag at which r drops below 0.50 is approximately the HINDCAST_LEN lower bound

## Step 2: Rolling Precipitation Cross-Correlation

Compute the Pearson correlation between a rolling mean of daily precipitation and the same-day water level, for rolling windows of 1 to 60 days. This answers: how long does rain continue to affect water level after it falls? This is the basin routing time.

# Python code to compute rolling precip cross-correlation for w in [1, 2, 3, 5, 7, 10, 14, 21, 30, 45, 60]: roll = df['tp'].rolling(w, min_periods=1).mean() tmp = pd.concat([roll, df['water_level']], axis=1).dropna() r, _ = pearsonr(tmp['tp'], tmp['water_level']) print(f'tp_roll{w:>2}d : r = {r:.4f}')

### Results for Brahmaputra-Jamuna (from your data)

| Rolling Window | Pearson r | Interpretation |
| --- | --- | --- |
| tp_roll1d | r = 0.581 | Weak — single day rain alone explains little WL |
| tp_roll3d | r = 0.649 | Moderate |
| tp_roll7d | r = 0.760 | Good |
| tp_roll14d | r = 0.853 | Strong |
| tp_roll21d | r = 0.887 | Strong |
| tp_roll30d | r = 0.911 | Very strong |
| tp_roll45d | r = 0.934 | Very strong — near peak |
| tp_roll60d | r = 0.943 | Peak — correlation plateaus here |

The correlation continues growing up to 60 days, which means rain from 60 days ago is still affecting the water level today. This directly confirms the large-basin routing lag and tells us that rolling windows up to 60 days are physically meaningful. Windows at 3, 7, 14, 30, and 60 days are chosen to sample this increasing correlation at regular intervals.

### How the plateau determines HINDCAST_LEN

Rule: HINDCAST_LEN = 1.5 × lag at which rolling precipitation correlation plateaus Brahmaputra: correlation plateaus at ~60 days → HINDCAST_LEN = 90 days Feni/Muhuri: correlation expected to plateau at ~7-14 days → HINDCAST_LEN = 30 days The 1.5× safety factor ensures the LSTM can see the full routing signal plus additional context for the rising and falling limb. Reference: Kratzert et al. (2019) showed that LSTM performance for large basins plateaus when the input sequence length covers the basin concentration time.

Step 3: Temperature Lag Analysis (Snowmelt vs ET)

Compute the cross-correlation between lagged daily temperature and water level. This identifies whether temperature influences WL through snowmelt routing (large lag, ~14-30 days) or only through evapotranspiration control on soil moisture (short lag, ~3-7 days).

# Python: best lag for temperature → WL correlation best_r, best_lag = 0, 0 for lag in range(0, 31): tmp = pd.concat([df['t2m'].shift(lag), df['water_level']], axis=1).dropna() r, _ = pearsonr(tmp['t2m'], tmp['water_level']) if abs(r) > abs(best_r): best_r, best_lag = r, lag print(f'Best r = {best_r:.4f} at lag = {best_lag} days')

### Results for Brahmaputra-Jamuna

| Best lag found | 15 days (r = 0.921) |
| --- | --- |
| Interpretation | Temperature ~15 days ago best predicts today's WL |
| Physical mechanism | Snowmelt on Tibetan Plateau: warm spell → melt → ~14-day routing to gauge |
| Conclusion | Include t2m_roll7, t2m_roll14, t2m_roll30 as features |

### How to interpret for any basin

| Observation | Diagnosis | Feature Window Decision |
| --- | --- | --- |
| Best lag > 7 days | Snowmelt/glacier contribution | Use t2m windows up to 30 days |
| Best lag 3-7 days | Mixed ET + warm-rain runoff | Use t2m windows 3-14 days |
| Best lag 0-2 days | ET control only (tropical) | Use short t2m windows (3, 7 days) |
| Mean t2m < 5°C | High altitude, glacier likely | Strong case for long t2m lags |
| Mean t2m > 20°C | Tropical — no snowmelt | Short t2m lags only (3, 7 days) |

For the Brahmaputra, the mean temperature in the ERA5 basin average was approximately 2°C, reflecting the dominance of high-altitude areas. For Muhuri/Feni at 21°C mean temperature, snowmelt is impossible and temperature acts primarily through evapotranspiration, justifying only short t2m rolling windows.

## Step 4: Basin Physics Assessment

The data analysis in Steps 1-3 must be cross-checked against basin physics. Some correlations in data may be spurious (driven by monsoon seasonality rather than true routing), and physical reasoning provides a sanity check.

### Key physical characteristics to assess

| Characteristic | How to measure | Brahmaputra | Feni/Muhuri |
| --- | --- | --- | --- |
| Basin area | From GIS shapefile | ~580,000 km² | ~2,770 km² |
| Time of concentration | Rational method / GIS analysis | ~30-60 days | ~12-24 hours |
| Channel routing time | Flow velocity × distance | ~45-60 days to gauge | ~3-7 days to gauge |
| Terrain gradient | DEM analysis | Plateau → plain | Hills → coastal plain |
| Snowmelt contribution | Mean basin elevation + t2m | Major (>4000 m MSL) | None (tropical) |
| Flood response type | Hydrograph shape | Slow, sustained | Fast, flashy |
| Annual rainfall | ERA5 tp mean × 365 | ~2,200 mm/yr | ~2,500 mm/yr |
| Monsoon fraction | Jun-Oct rainfall / annual | ~70% | ~85% |

### Time of Concentration (Tc) estimation

The time of concentration is the time for water from the most distant point of the catchment to reach the outlet. It is the primary determinant of HINDCAST_LEN and rolling window sizes.

Kirpich Formula (simplified) — for small to medium basins Tc (hours) = 0.0195 * L0.77 × S0.385 where: L = length of longest flow path (m) S = average slope (m/m) For Muhuri: L ≈ 80,000 m, S ≈ 0.003 → Tc ≈ 14 hours For Brahmaputra main stem: L ≈ 2,900,000 m, S ≈ 0.0001 → Tc ≈ 8 days Note: Tc gives the MINIMUM routing time. Total lag to gauge is typically 3-5× Tc for large basins. Reference: Kirpich (1940), USDA TR-55 (1986).

## Step 5: Setting HINDCAST_LEN

HINDCAST_LEN is the number of past days the LSTM encoder reads as its input window. It is the most critical hyperparameter because it determines how much historical context the model has access to. Too short: the model misses the slow routing signal. Too long: the LSTM is diluted with irrelevant ancient history and training becomes slower.

### Decision Rule

HINDCAST_LEN = max (rolling_precip_plateau_lag, WL_memory_lag) × safety_factor Step 2 plateau lag: the rolling window size at which cross-correlation stops improving significantly WL memory lag: the lag at which WL autocorrelation drops below 0.50 (from Step 1) Safety factor: typically, 1.3 to 1.5 to capture rising/falling limb context beyond the routing signal Brahmaputra: max (60 days, ~65 days) × 1.5 → rounded to 90 days Feni/Muhuri: max (14 days, ~20 days) × 1.5 → rounded to 30 days

### HINDCAST_LEN Reference Table

| Basin Type | Area | Routing Lag | HINDCAST_LEN |
| --- | --- | --- | --- |
| Basin type | Basin area | Typical routing lag | Recommended HINDCAST_LEN |
| Small flashy | < 500 km² | 6-24 hours | 7-14 days |
| Small monsoon | 500-5,000 km² | 1-7 days | 14-30 days |
| Medium basin | 5,000-50,000 km² | 7-30 days | 30-60 days |
| Large basin | 50,000-500,000 km² | 30-90 days | 60-120 days |
| Very large (Brahma) | > 500,000 km² | 45-90 days | 90-180 days |

Note: these are starting points. Always verify against your Step 1 and Step 2 results. The data always takes precedence over the rule of thumb table.

## Step 6: Feature Window Selection

Features are engineered from the raw ERA5 variables (tp, t2m, ssr, str, sp) using rolling means, lags, and derived indices. Every window size must be justified by the analysis from Steps 1-4.

### 6.1 Rolling Precipitation Windows

The rolling precipitation windows capture the accumulated upstream inflow that is still routing through the catchment. The rule is: include windows that correspond to meaningful steps in the cross-correlation curve from Step 2.

| Window | Physical Meaning | When to Include |
| --- | --- | --- |
| Window | Physical meaning | When to include |
| 1-2 day | Same-day and previous-day rainfall | Always — captures immediate response |
| 3 days | Short-duration storm total | Always — typical storm duration |
| 5-7 day | Weekly antecedent moisture | Always — whether soil is wet or dry |
| 10-14 day | Recent wet/dry spell state | Include if correlation plateau > 10 days |
| 21-30 day | Monthly antecedent moisture for large basins | Include if correlation plateau > 20 days |
| 45-60 day | Long-range routing for very large/glaciated basins | Only if plateau > 40 days (e.g. Brahmaputra) |

Practical rule: pick 5-6 windows that sample the correlation curve at roughly equal intervals from 2 days to the plateau lag. Do not include consecutive windows (e.g. 7d, 8d, 9d) — adjacent windows are highly correlated and add no new information (multicollinearity).

### 6.2 Rolling Temperature Windows

Temperature influences WL through two distinct mechanisms. Identify which mechanism dominates before choosing windows.

| Mechanism 1: Snowmelt routing | Cold high-elevation catchments. Warm spell → glacier/snow melt → meltwater routes downstream with a lag of 7-30 days. Best lag from Step 3 will be > 7 days. Include t2m windows of 7, 14, and 30 days. Reference: Immerzeel et al. (2010) — Himalayan glaciers contribution to river flow. |
| --- | --- |
| Mechanism 2: ET control | Tropical/subtropical basins with no snow. Warm day → high evapotranspiration → drier soil → less runoff when next rain arrives. This lag is short (3-5 days) and the correlation is weaker. Include only t2m windows of 3 and 7 days. Reference: Allen et al. (1998) FAO-56 reference evapotranspiration. |
| Decision criterion | If mean basin temperature < 5°C OR Step 3 best lag > 10 days → Mechanism 1 dominant → use longer windows (7, 14, 30 days). If mean temperature > 15°C AND Step 3 best lag < 7 days → Mechanism 2 dominant → use shorter windows (3, 7 days only). |

### 6.3 Autoregressive Water Level Lags

The WL lags are the most powerful features in the model (r > 0.99 for lag-1 in your data). They encode the integrated catchment state — soil moisture, channel storage, groundwater — in a single variable that no meteorological input can match. The autocorrelation analysis from Step 1 determines which lags to include.

Selection rule: include WL lags at days where the autocorrelation from Step 1 is still > 0.80. Space them to avoid multicollinearity — do not include every consecutive lag. Brahmaputra (autocorr still > 0.80 at lag-30): lags 1, 2, 3, 7, 14 Feni/Muhuri (autocorr expected < 0.80 by lag-10): lags 1, 2, 3, 5, 7 Always include lag-1 and lag-2 (near-perfect predictors). Include lag-7 if basin has weekly monsoon cycles. Include lag-14 only for large basins with slow routing.

### 6.4 WL Anomaly Window

The WL anomaly (current WL minus rolling mean WL) tells the model whether the river is currently rising or falling. This is critical for correct peak timing and hydrograph shape.

| Formula | wl_anom = WL (today) - rolling_mean(WL, window) |
| --- | --- |
| Positive value | River is above its recent average → rising limb → flood building |
| Negative value | River is below its recent average → falling limb → recession |
| Window for Brahmaputra | 30 days — flood season lasts 3-4 months, baseline shifts slowly |
| Window for Muhuri | 7 days — flash floods last 1-3 days, need fast-responding anomaly |
| General rule | Window ≈ 0.5 × HINDCAST_LEN, minimum 7 days |

### 6.5 Rain × Temperature Interaction

The product of rolling precipitation and rolling temperature captures a physical non-linearity: warm rainfall generates runoff much faster than cold-season precipitation (Penman, 1948; Bergström, 1992). Cold rain is more likely to freeze or be absorbed by cold dry soil; warm rain hits warm saturated soil and generates immediate surface runoff.

# Warm-rain interaction feature df['tp_x_t2m'] = df['tp_roll3'] * df['t2m_roll3'] # High value: warm, wet conditions → rapid runoff # Low value: cold or dry conditions → slower/less runoff

This single feature can capture the difference between a 50 mm monsoon rain event (warm soil, rapid runoff, fast flood peak) and a 50 mm winter rain event in high-altitude areas (cold soil, slow infiltration, delayed peak).

### 6.6 Cyclical Temporal Encoding

Day-of-year and month are encoded as sine/cosine pairs rather than raw numbers. This ensures there is no discontinuity at year boundaries (December 31 to January 1 is a small step, not a jump from 365 to 1) and that the model can learn smooth seasonal patterns.

doy = df.index.dayofyear df['doy_sin'] = sin (2π × doy / 365.25) df['doy_cos'] = cos (2π × doy / 365.25) df['month_sin'] = sin (2π × month / 12) df['month_cos'] = cos (2π × month / 12)

Reference: This encoding approach is standard in time-series deep learning. Vaswani et al. (2017) used identical positional encoding in the Transformer architecture. For river forecasting, it allows the LSTM to learn that July (monsoon peak) is consistently different from January (dry season) without requiring the model to learn the year-boundary discontinuity.

## Step 7: Model Architecture Decisions

### 7.1 HIDDEN_SIZE

HIDDEN_SIZE controls the number of memory cells in the LSTM layer. It determines the model's capacity to represent complex non-linear relationships. The key trade-off is: larger = more expressive but higher overfitting risk on small datasets.

| Factor | Brahmaputra | Feni/Muhuri | General principle |
| --- | --- | --- | --- |
| Basin complexity | Very high (snow + rain + GW) | Low (rain only) | More complex → larger HIDDEN_SIZE |
| Number of processes | 4+ (glacier, monsoon, GW, routing) | 1 (monsoon rain) | More processes → more capacity needed |
| Training set size | ~10,400 sequences | ~10,400 sequences | More data → can support larger model |
| Feature correlation | Moderate (diverse signal) | Very high (wl_lag1 r=0.999) | High r → simple model is sufficient |
| HIDDEN_SIZE chosen | 128 | 64 | Scale to complexity |

Practical guidance on HIDDEN_SIZE 32-64: Small simple basins (< 5,000 km²), one dominant process (monsoon only) 64-128: Medium basins, 2-3 processes, moderate data 128-256: Large complex basins (Brahmaputra, Ganges), multiple processes, large datasets 256+: Multi-basin transfer learning, global models (as in Nearing et al. 2024) If validation loss is much higher than training loss → model too large → reduce HIDDEN_SIZE or increase DROPOUT If both training and validation losses are high → model too small → increase HIDDEN_SIZE Reference: Kratzert et al. (2018) found hidden sizes of 128-256 optimal for large-sample hydrology.

### 7.2 DROPOUT

Dropout randomly zeroes a fraction of neurons during each training step, forcing the network to learn redundant representations and preventing co-adaptation. This is the primary regularization mechanism.

| Why 0.25 for Muhuri, not 0.20? | The Muhuri signal is dominated by wl_lag1 (r = 0.999). Without sufficient dropout, the model would learn to simply copy wl_lag1 and assign near-zero weight to precipitation and temperature features. Higher dropout forces the model to learn from all features, including the routing information in tp_roll3, tp_roll5, and wl_anom_7d. |
| --- | --- |
| Why 0.20 for Brahmaputra? | The Brahmaputra signal is more distributed across features. wl_lag1 is still strong (r = 0.999) but t2m_roll14 (r = 0.92), tp_roll60 (r = 0.94), and wl_anom_30d all carry significant independent signal. The model naturally distributes attention across features, so lower dropout is sufficient. |
| General guideline | Start at 0.20. If feature correlation plot shows one feature with r > 0.99 and all others < 0.80, increase to 0.25-0.30 to prevent the model from ignoring non-dominant features. |
| Upper limit | Do not exceed 0.40 dropout for LSTM models. Very high dropout destroys the temporal memory that is the LSTM's primary advantage. Reference: Gal & Ghahramani (2016) showed that dropout rates > 0.50 degrade LSTM sequential memory. |

### 7.3 N_LAYERS (LSTM depth)

All models in this framework use a single LSTM layer (N_LAYERS = 1). This is a deliberate choice, not a limitation.

Why single-layer LSTM for hydrological modelling? 1. Kratzert et al. (2019) found that single-layer LSTM outperformed 2-layer LSTM on the CAMELS benchmark for 531 US catchments. 2. The physics of basin routing is a first-order differential equation (dS/dt = Qin - Qout). A single LSTM layer is sufficient to represent this dynamic. Stacked layers add capacity to learn complex non-linear spatial patterns, which is not the primary challenge here. 3. In our single-basin setting, the input already contains the routing physics (wl_lag features, rolling windows). The LSTM's job is integration, not feature extraction. One layer is sufficient. When to use 2 layers: multi-basin transfer learning, when input is raw meteorological data without engineered features, or when single-layer NSE < 0.80 and data size is very large (> 50,000 sequences).

## Step 8: Training Hyperparameters

### 8.1 Learning Rate (LR = 5×10⁻⁴)

The learning rate controls the step size in gradient descent. Too large: training oscillates and diverges. Too small: training is unnecessarily slow and may get stuck in local minima.

| Value chosen | 5×10⁻⁴ (= 0.0005) |
| --- | --- |
| Standard range | 1×10⁻⁴ to 1×10⁻³ for LSTM regression tasks |
| Why not 1×10⁻³? | Adam with LR=0.001 can overshoot on smooth hydrological loss surfaces, especially in the first few epochs when all WL lags are present |
| Why not 1×10⁻⁴? | Unnecessarily slow convergence on ~10,000 training sequences with batch size 256 |
| LR scheduling | ReduceLROnPlateau with factor=0.5, patience=8 — halves LR if validation loss does not improve for 8 epochs. This allows fast initial learning and fine precision near convergence. |
| Reference | Kingma & Ba (2015) Adam optimizer. LR range testing by Smith (2017) suggests 5×10⁻⁴ as a robust default for regression on geophysical data. |

### 8.2 PATIENCE = 25

Early stopping halts training when the validation loss does not improve for PATIENCE consecutive epochs, preventing overfitting.

| Why 25 epochs? | With the LR scheduler reducing LR every 8 epochs of no improvement, there are potentially 3 LR reductions (at epochs 8, 16, 24 of stagnation) before early stopping triggers. This allows the model to escape local plateaus after each LR reduction. |
| --- | --- |
| If training stops too early | Validation loss may still be improving very slowly. Increase PATIENCE to 35-40. Check the training curve plot (01_training_curves.png) — if the validation line is still decreasing at the stopping point, increase PATIENCE. |
| If training takes too long | Reduce PATIENCE to 15. On smaller basins with simple dynamics, 15 epochs without improvement genuinely indicates convergence. |

### 8.3 BATCH_SIZE = 256

| Why 256? | Balances computational efficiency with gradient quality. On ~10,000 training sequences, batch 256 gives ~40 gradient updates per epoch — sufficient for stable convergence. |
| --- | --- |
| Effect on performance | Larger batches (512+) can degrade generalization slightly (Keskar et al., 2017) — sharp minima. Smaller batches (32-64) add noise that helps regularization but slows training. |
| Memory constraint | 256 × 30 × 24 = 184,320 float32 values per batch = ~700 KB. Well within CPU/GPU memory for any modern system. |

### 8.4 GRADIENT CLIP = 1.0

| Purpose | Prevents exploding gradients — a known instability in RNN/LSTM training (Pascanu et al., 2013). The gradient norm is scaled to 1.0 if it exceeds this threshold. |
| --- | --- |
| Why 1.0? | Standard for LSTM regression tasks. Clipping at 1.0 eliminates catastrophic gradient spikes while allowing normal parameter updates. Kratzert et al. (2019) used the same value for their large-sample hydrology LSTM. |
| Effect | Rarely activates during stable training. If it activates frequently (loss is volatile), reduce LR instead. |

# 3.  Basin Comparison Reference Table

This table summarizes all parameter decisions for the two basins modelled in this project, with the physical justification for each difference.

| Parameter | Brahmaputra-Jamuna | Feni / Muhuri | Why different |
| --- | --- | --- | --- |
| HINDCAST_LEN | 90 days | 30 days | Routing lag 60d vs 7d |
| tp_roll windows | 3,7,14,30,60 days | 2,3,5,7,10,14 days | Plateau at 60d vs 14d |
| t2m_roll windows | 7,14,30 days | 3,7 days | Snowmelt vs ET only |
| WL lags | 1,2,3,7,14 days | 1,2,3,5,7 days | Memory 14d vs 7d |
| WL anomaly window | 30 days | 7 days | Slow vs fast limb |
| HIDDEN_SIZE | 128 | 64 | Complex vs simple process |
| DROPOUT | 0.20 | 0.25 | Prevent wl_lag1 dominance |
| Total features | 24 | 24 | Same count, different windows |
| Mean t2m | ~2°C | ~21°C | Snowmelt possible vs impossible |
| tp_roll60 useful? | Yes (r=0.94) | No (noise) | Basin routing time |
| tp cross-corr peak | ~60 days | ~14 days | Basin size and routing time |

# 4.  Diagnostic Workflow for a New Basin

When applying this model to any new river basin, follow this sequential diagnostic workflow. The outputs of earlier steps directly feed into later decisions.

## 4.1 Mandatory Analysis Checklist

Run this Python analysis script before writing any model code from scipy.stats import pearsonr import pandas as pd, numpy as np # Step 1: WL autocorrelation wl = df['water_level'].dropna() for lag in [1,2,3,5,7,10,14,21,30,60]: r,_=pearsonr(wl[lag:].values,wl[:-lag].values) print(f'WL autocorr lag{lag:>2}d: r={r:.4f}') # Step 2: Rolling precip cross-correlation for w in [1,2,3,5,7,10,14,21,30,45,60]: roll=df['tp'].rolling(w,min_periods=1).mean() tmp=pd.concat([roll,df['water_level']],axis=1).dropna() r,_=pearsonr(tmp['tp'],tmp['water_level']) print(f'tp_roll{w:>2}d vs WL: r={r:.4f}') # Step 3: t2m best lag best_r,best_lag=0,0 for lag in range(0,31): tmp=pd.concat([df['t2m'].shift(lag),df['water_level']],axis=1).dropna() r,_=pearsonr(tmp['t2m'],tmp['water_level']) if abs(r)>abs(best_r): best_r,best_lag=r,lag print(f't2m best lag: {best_lag}d, r={best_r:.4f}') print(f't2m mean: {df.t2m.mean():.1f} C')

## 4.2 Decision Flowchart

Follow this logic tree to set HINDCAST_LEN and feature windows:

| Q1: Where does rolling precip correlation plateau? | Answer → sets HINDCAST_LEN upper bound |
| --- | --- |
| Plateau at < 14 days (small fast basin) | → HINDCAST_LEN = 21-30 days. Use tp windows 2,3,5,7,10,14d |
| Plateau at 14-30 days (medium basin) | → HINDCAST_LEN = 30-45 days. Use tp windows 3,5,7,14,21,30d |
| Plateau at 30-60 days (large basin) | → HINDCAST_LEN = 60-90 days. Use tp windows 7,14,21,30,45,60d |
| Q2: What is mean basin temperature? | Answer → sets t2m window length |
| Mean t2m < 5°C OR t2m best lag > 10 days | → Snowmelt dominant. Use t2m_roll7, t2m_roll14, t2m_roll30 |
| Mean t2m 5-15°C | → Mixed. Use t2m_roll3, t2m_roll7, t2m_roll14 |
| Mean t2m > 15°C | → ET only. Use t2m_roll3, t2m_roll7 only |
| Q3: At what lag does WL autocorr drop below 0.90? | Answer → sets max WL lag |
| WL autocorr < 0.90 at lag-7 (flashy basin) | → Use lags 1, 2, 3, 5, 7 |
| WL autocorr > 0.90 at lag-14 (large basin) | → Use lags 1, 2, 3, 7, 14 |
| WL autocorr > 0.90 at lag-21 (very large) | → Use lags 1, 2, 3, 7, 14, 21 |
| Q4: How does the basin respond to rain? | Answer → sets wl_anom window |
| Flash floods, peak in < 3 days | → wl_anom_7d |
| Moderate response, peak in 3-10 days | → wl_anom_14d |
| Slow response, seasonal flooding | → wl_anom_30d |

## 4.3 Validation Checks

After training, use these plots and metrics to verify the parameter decisions were correct.

| Diagnostic | What to look for | Red flag | Fix |
| --- | --- | --- | --- |
| Plot / metric | What to check | If failing | Corrective action |
| 05_feature_correlation.png | wl_lag1 should be top feature. For small basins, tp_roll3-7 should rank higher than tp_roll30-60 | tp_roll60 ranks high for a small basin | Your basin may be larger than assumed. Re-check basin area and routing analysis. |
| 06_lag_analysis.png | Rolling precip plateau should match chosen HINDCAST_LEN | Plateau is at 20d but HINDCAST_LEN=90d | Reduce HINDCAST_LEN to 30d and retrain. |
| 01_training_curves.png | Val loss should track train loss closely with no large gap | Large gap (val >> train) | Overfitting. Increase DROPOUT or reduce HIDDEN_SIZE. |
| Test NSE | Should be > 0.85 for well-gauged single basins | NSE < 0.70 | Check missing data, feature scaling, or increase HINDCAST_LEN if routing signal is longer than assumed. |
| 04_seasonal_error.png | Errors should be similar across months | High error in Jun-Sep only | Model missing monsoon peak signal. Check tp_roll window adequacy. |

# 5.  References

| Nearing et al. (2024) | Global prediction of extreme floods in ungauged watersheds. Nature, 627, 559-563. https://doi.org/10.1038/s41586-024-07145-1. [Primary reference for the LSTM architecture and encoder-decoder design used in this project.] |
| --- | --- |
| Kratzert et al. (2019a) | Towards learning universal, regional, and local hydrological behaviors via machine learning applied to large-sample datasets. Hydrology and Earth System Sciences, 23, 5089-5110. [Established LSTM as state-of-the-art for rainfall-runoff modelling; justifies single-layer architecture and HIDDEN_SIZE=128 for complex basins.] |
| Kratzert et al. (2019b) | Toward improved predictions in ungauged basins: Exploiting the power of machine learning. Water Resources Research, 55, 11344-11354. [Demonstrated LSTM transfer learning to ungauged basins; basis for the WL lag feature selection approach.] |
| Hochreiter & Schmidhuber (1997) | Long short-term memory. Neural Computation, 9(8), 1735-1780. [Original LSTM paper. The gated memory cell architecture is the core of all models in this project.] |
| Kingma & Ba (2015) | Adam: A method for stochastic optimization. ICLR 2015. [Justification for Adam optimizer with LR=5e-4. Adam's adaptive learning rates are particularly suited to the heterogeneous feature scales in hydrological inputs.] |
| Pascanu et al. (2013) | On the difficulty of training recurrent neural networks. ICML 2013. [Theoretical basis for gradient clipping at norm=1.0. Establishes that exploding gradients are an intrinsic property of deep RNNs and clipping is the standard remedy.] |
| Gal & Ghahramani (2016) | A theoretically grounded application of dropout in recurrent neural networks. NIPS 2016. [Basis for DROPOUT rate selection. Shows that dropout > 0.40 degrades LSTM sequential memory; justifies 0.20-0.25 range used here.] |
| Sivapalan et al. (2003) | IAHS Decade on Predictions in Ungauged Basins (PUB), 2003-2012. Hydrological Sciences Journal, 48(6), 857-880. [Context for the ungauged basin problem; motivates the transfer-learning approach of the LSTM.] |
| Immerzeel et al. (2010) | Climate change will affect the Asian water towers. Science, 328, 1382-1385. [Quantified glacier and snowmelt contribution to Himalayan rivers; basis for including 14-30 day t2m rolling windows for Brahmaputra.] |
| Allen et al. (1998) | Crop evapotranspiration: Guidelines for computing crop water requirements. FAO Irrigation and Drainage Paper 56. [Basis for short t2m windows (3-7 days) for ET-controlled tropical basins like Muhuri/Feni.] |
| Beven (2012) | Rainfall-Runoff Modelling: The Primer, 2nd ed. Wiley-Blackwell. [Chapter 2: basin concentration time and routing lag concepts. Theoretical basis for HINDCAST_LEN = 1.5 × routing lag rule.] |
| Kirpich (1940) | Time of concentration of small agricultural watersheds. Civil Engineering, 10(6), 362. [Original formula for estimating time of concentration from basin geometry; used for Feni/Muhuri HINDCAST_LEN estimation.] |
| Keskar et al. (2017) | On large-batch training for deep learning: Generalization gap and sharp minima. ICLR 2017. [Justification for BATCH_SIZE=256: shows that larger batches converge to sharp minima with worse generalization.] |
| Boulmaiz et al. (2020) | Impact of training data size on the LSTM performances for rainfall-runoff modeling. Modeling Earth Systems and Environment, 6, 2153-2164. [Showed that single-step LSTM with autoregressive lags outperforms multi-step encoder-decoder for peak discharge prediction in small basins.] |
| USDA (1986) | Urban hydrology for small watersheds. Technical Release 55 (TR-55). USDA Natural Resources Conservation Service. [Runoff curve number method and time of concentration equations used for small basin characterization.] |

# Appendix: Quick Reference Card

This card summarises the complete decision framework on one page for field reference.

Step 1 → Step 2 → Step 3 → Step 4 → Step 5 → Step 6 → Step 7 → Step 8 Run all 8 steps in order before writing any model code.

| Parameter | Decision basis | Brahmaputra | Feni/Muhuri |
| --- | --- | --- | --- |
| Decision | How to determine | Brahmaputra | Feni/Muhuri |
| HINDCAST_LEN | 1.5 × rolling precip plateau lag | 90 days | 30 days |
| tp_roll windows | Sample corr curve, avoid adjacent | 3,7,14,30,60d | 2,3,5,7,10,14d |
| t2m_roll windows | Snowmelt: long. ET only: short | 7,14,30d | 3,7d |
| WL lags | Include where autocorr > 0.80 | 1,2,3,7,14d | 1,2,3,5,7d |
| wl_anom window | ≈ 0.5 × HINDCAST_LEN | 30d | 7d |
| HIDDEN_SIZE | Complexity: small=64, large=128+ | 128 | 64 |
| DROPOUT | 0.20 base; +0.05 if lag1 r>0.99 | 0.20 | 0.25 |
| LR | 5×10⁻⁴ default for LSTM regression | 5×10⁻⁴ | 5×10⁻⁴ |
| PATIENCE | 3 × LR_scheduler_patience | 25 | 25 |
| BATCH_SIZE | 256 defaults for ~10k sequences | 256 | 256 |
| GRAD_CLIP | 1.0 — standard for LSTM | 1.0 | 1.0 |
| N_LAYERS | 1 for single basin; 2 for transfer | 1 | 1 |

End of Technical Manual | Version 1.0 | 2025